A fuzzy logic controller for PLC in SCL (ST)

Fuzzy Logic Controller Memberships

Step Response

Transfer function of the Fuzzy Controller

Listing 1 SCL Example:

FUNCTION_BLOCK gauss_membership_leftopen

VAR_INPUT

x : REAL;

mean : REAL;

sigma : REAL;

END_VAR

VAR_OUTPUT

membership_value : REAL;

END_VAR

IF x < mean THEN

membership_value := 1.0;

ELSE

membership_value := EXP(-0.5 * POWER((x - mean) / sigma, 2));

END_IF

END_FUNCTION_BLOCK

FUNCTION_BLOCK gauss_membership

VAR_INPUT

x : REAL;

mean : REAL;

sigma : REAL;

END_VAR

VAR_OUTPUT

membership_value : REAL;

END_VAR

membership_value := EXP(-0.5 * POWER((x - mean) / sigma, 2));

END_FUNCTION_BLOCK

FUNCTION_BLOCK gauss_membership_rightopen

VAR_INPUT

x : REAL;

mean : REAL;

sigma : REAL;

END_VAR

VAR_OUTPUT

membership_value : REAL;

END_VAR

IF x > mean THEN

membership_value := 1.0;

ELSE

membership_value := EXP(-0.5 * POWER((x - mean) / sigma, 2));

END_IF

END_FUNCTION_BLOCK

FUNCTION PT2_Simulation : REAL

VAR_INPUT

ControlInput : REAL;

END_VAR

VAR

deltaT : REAL := 1.0;

k1 : REAL := 82.0 / 100;

k2 : REAL := 1.0;

k3 : REAL := 1.0;

T1 : REAL := 100.0;

T2 : REAL := 8.0;

T3 : REAL := 10.0;

x1_k : REAL := 0;

x2_k : REAL := 0;

x3_k : REAL := 0;

x1_dot : REAL;

x2_dot : REAL;

x3_dot : REAL;

END_VAR

x1_dot := (k1 * ControlInput - x1_k) / T1;

x1_k := x1_dot * deltaT + x1_k;

x2_dot := (k2 * x1_k - x2_k) / T2;

x2_k := x2_dot * deltaT + x2_k;

x3_dot := (k3 * x2_k - x3_k) / T3;

x3_k := x3_dot * deltaT + x3_k;

PT2_Simulation := x3_k;

END_FUNCTION

FUNCTION fuzzy_control : REAL

VAR_INPUT

e : REAL;

mean_left : REAL;

sigma_left : REAL;

mean_mid : REAL;

sigma_mid : REAL;

mean_right : REAL;

sigma_right : REAL;

END_VAR

VAR

left : REAL;

middle : REAL;

right : REAL;

numerator : REAL;

denominator : REAL;

y : REAL;

END_VAR

left := gauss_membership_leftopen(e, mean_left, sigma_left);

middle := gauss_membership(e, mean_mid, sigma_mid);

right := gauss_membership_rightopen(e, mean_right, sigma_right);

numerator := left * -70.0 + middle * 0 + right * 70.0;

denominator := left + middle + right;

IF denominator <> 0 THEN

y := numerator / denominator;

ELSE

y := 0;

END_IF

fuzzy_control := y;

END_FUNCTION

PROGRAM Main

VAR

loop : INT := 0;       // Loop counter

w : REAL := 30.0;      // Setpoint or desired output

y : REAL := 0.0;       // Output of the system

x : REAL := 0.0;       // Input to the system (state variable)

e : REAL := 0.0;       // Error signal

factor : REAL;         // Difference between the setpoint and the final system output

END_VAR

// Loop for PT2 system simulation and fuzzy control

FOR loop := 0 TO 499 DO

// Simulate the PT2 system and calculate the error signal

x := PT2_Simulation(y);                // Simulate the PT2 system and update the state variable

e := w - x;                             // Calculate the error signal

// Apply fuzzy control to calculate the control signal

// Uncomment the following line to apply fuzzy control with a constant offset

y := fuzzy_control(e + 2.66);           // Apply fuzzy control with a constant offset

// y := fuzzy_control(e);                // Apply fuzzy control

// Print epoch information (iteration number) and system variables

// (Replace PRINT with appropriate function to output to PLC's diagnostic or HMI)

// PRINT("epoch = %d w = %.5f, e = %.5f, x = %.5f, y = %.5f", loop, w, e, x, y);

END_FOR

// Calculate the difference between the setpoint and the final system output

factor := w - x;

// Print the difference between the setpoint and the final system output

// (Replace PRINT with appropriate function to output to PLC's diagnostic or HMI)

// PRINT("factor = %.5f", factor);

// End of program

END_PROGRAM

Listing 2 Ansi C Example:

/*

Software: Fuzzy Test of Gaussian membership functions

Autor: Josef Bernhardt www.bernhardt.de

Date: 27.01.2023

Codeblocks ANSI C Enviroment for test

E-mail: josef@bernhardt.de for a complete control program

*/

#include <stdio.h>

#include <math.h>

// Mean and standard deviation for the left side of the x axis

float mean_left = -5;   // Mean value for the left side of the x axis

float sigma_left = 2.5; // Standard deviation for the left side of the x axis

// Mean and standard deviation for the middle of the x axis

float mean_mid = 0.0;   // Mean value for the middle of the x axis

float sigma_mid = 2.5;  // Standard deviation for the middle of the x axis

// Mean and standard deviation for the right side of the x axis

float mean_right = 5.0;   // Mean value for the right side of the x axis

float sigma_right = 2.5;  // Standard deviation for the right side of the x axis

// This function calculates the membership value of a given input

// 'x' in a left-open Gaussian membership function.

// The left-open Gaussian membership function returns 1.0 for

// input values less than the mean,

// and the standard Gaussian membership function value otherwise.

float gauss_membership_leftopen(float x, float mean, float sigma) {

if (x < mean) {

return 1.0; // Outside the interval, return 0

}

return exp(-0.5 * pow((x - mean) / sigma, 2));

}

// This function calculates the membership value of a given input 'x'

// in a standard Gaussian membership function.

// The standard Gaussian membership function is continuous and

// returns a value based on the input, mean, and standard deviation.

float gauss_membership(float x, float mean, float sigma) {

return exp(-0.5 * pow((x - mean) / sigma, 2));

}

// This function calculates the membership value of a given input 'x'

// in a right-open Gaussian membership function.

// The right-open Gaussian membership function returns 1.0 for

// input values greater than the mean,

// and the standard Gaussian membership function value otherwise.

float gauss_membership_rightopen(float x, float mean, float sigma) {

if (x > mean) {

return 1.0; // Outside the interval, return 0

}

return exp(-0.5 * pow((x - mean) / sigma, 2));

}

// This function calculates the control signal (y) based on the error signal (e) using fuzzy logic.

float fuzzy_control(float e) {

// Calculate the membership values for the error signal in different regions of the fuzzy sets.

float left   = gauss_membership_leftopen(e, mean_left, sigma_left);   // Membership value for the left region

float middle = gauss_membership(e, mean_mid, sigma_mid);              // Membership value for the middle region

float right  = gauss_membership_rightopen(e, mean_right, sigma_right);// Membership value for the right region

// Calculate the weighted sum of membership values to determine the control signal.

float numerator   = left * -70.0 + middle * 10 + right * 70.0; // Weighted sum of membership values

float denominator = left + middle + right;                    // Sum of membership values

float y; // Control signal

// Avoid division by zero by checking if the denominator is zero.

if (denominator != 0) {

// Calculate the control signal as the ratio of the weighted sum to the sum of membership values.

y = numerator / denominator;

} else {

// If the denominator is zero, set the control signal to zero to avoid undefined behavior.

y = 0;

}

// Uncomment the following line to print the membership values for debugging purposes.

// printf("The membership for e = %.5f, y = %.5f, left = %.5f middle = %.5f right = %.5f\n", e, y, left, middle, right);

return y; // Return the control signal.

}

// This function simulates a second-order system using a discrete-time approach (Euler integration).

float PT2_Simulation(float ControlInput) {

// Parameters

const float deltaT = 1.0;   // Time step (max 10)

const float k1 = 82.0 / 100; // Gain factor

const float k2 = 1.0;        // Gain factor

const float k3 = 1.0;        // Gain factor

const float T1 = 100.0;      // Time constant

const float T2 = 8.0;        // Time constant

const float T3 = 10.0;       // Time constant

// Variables

static float x1_k = 0; // State variable 1 at time k

static float x2_k = 0; // State variable 2 at time k

static float x3_k = 0; // State variable 3 at time k

static float x1_dot;   // Rate of change of state variable 1

static float x2_dot;   // Rate of change of state variable 2

static float x3_dot;   // Rate of change of state variable 3

// Model equations using Euler integration

x1_dot = (k1 * ControlInput - x1_k) / T1;   // Rate of change of state variable 1

x1_k = x1_dot * deltaT + x1_k;              // Update state variable 1 using Euler integration

x2_dot = (k2 * x1_k - x2_k) / T2;           // Rate of change of state variable 2

x2_k = x2_dot * deltaT + x2_k;              // Update state variable 2 using Euler integration

x3_dot = (k3 * x2_k - x3_k) / T3;           // Rate of change of state variable 3

x3_k = x3_dot * deltaT + x3_k;              // Update state variable 3 using Euler integration

return x3_k;                                // Return the output of the system (state variable 3 at time k)

}

// Main function to demonstrate the fuzzy controller simulation and PT2 system simulation.

int main() {

// Dateinamen festlegen

char *filename = "ascii_values.csv";

// Datei zum Schreiben öffnen

FILE *file = fopen(filename, "w");

// Überprüfen, ob die Datei erfolgreich geöffnet wurde

if (file == NULL) {

printf("Fehler beim Öffnen der Datei.");

return 1;

}

int loop = 0; // Loop counter

float w = 30.0; // Setpoint or desired output

float y = 0;    // Output of the system

float x = 0;    // Input to the system (state variable)

float e = 0;    // Error signal

// fprintf(file,"w x y");

// Loop for PT2 system simulation and fuzzy control

for (loop = 0; loop < 600; loop++) {

// Simulate the PT2 system and calculate the error signal

x = PT2_Simulation(y); // Simulate the PT2 system and update the state variable

e = w - x;              // Calculate the error signal

// Apply fuzzy control to calculate the control signal

// Uncomment the following line to apply fuzzy control with a constant offset

y = fuzzy_control(e +2.32); // Apply fuzzy control with a constant offset

// y = fuzzy_control(e); // Apply fuzzy control

// Print epoch information (iteration number) and system variables

printf("epoch = %d w = %.5f, e = %.5f, x = %.5f, y = %.5f \n", loop, w, e, x, y);

// fprintf(file,"epoch = %d w = %.5f, e = %.5f, x = %.5f, y = %.5f \n", loop, w, e, x, y);

fprintf(file,"%.5f %.5f %.5f \n", w, x, y);

}

// Print the difference between the setpoint and the final system output

printf("e = %.5f",e);

// system("pause");

printf("Controller characteristic curve\n");

printf("float trainingPatterns[MAX_PATTERNS][MAX_INPUT_NEURONS] = {");

for (float ll = -10; ll < 11; ll+=1.0)

{

y = fuzzy_control(ll);

printf("{%.5f} , ",ll);

}

printf("};\n");

printf("float targetOutputs[MAX_PATTERNS][MAX_OUTPUT_NEURONS] = {");

for (float ll = -10; ll < 11; ll+=1.0)

{

y = fuzzy_control(ll);

printf("{%.7f}, ",y/100.0);

}

printf("};\n");

// Datei schließen

fclose(file);

system("pause");

return 0; // Return 0 to indicate successful execution

}

/*

Controller characteristic curve

e = -10.000, y = -99.966

e = -9.000, y = -99.847

e = -8.000, y = -99.406

e = -7.000, y = -98.053

e = -6.000, y = -94.673

e = -5.000, y = -88.024

e = -4.000, y = -76.627

e = -3.000, y = -59.085

e = -2.000, y = -37.876

e = -1.000, y = -17.649

e = 0.000, y = 0.000

e = 1.000, y = 17.649

e = 2.000, y = 37.876

e = 3.000, y = 59.085

e = 4.000, y = 76.627

e = 5.000, y = 88.024

e = 6.000, y = 94.673

e = 7.000, y = 98.053

e = 8.000, y = 99.406

e = 9.000, y = 99.847

e = 10.000, y = 99.966

example loop

epoch = 472 w = 30.00000, e = 0.00992, x = 29.99008, y = 36.58100

epoch = 473 w = 30.00000, e = 0.00999, x = 29.99001, y = 36.58207

epoch = 474 w = 30.00000, e = 0.01005, x = 29.98995, y = 36.58303

epoch = 475 w = 30.00000, e = 0.01011, x = 29.98989, y = 36.58385

epoch = 476 w = 30.00000, e = 0.01016, x = 29.98984, y = 36.58455

epoch = 477 w = 30.00000, e = 0.01020, x = 29.98980, y = 36.58515

epoch = 478 w = 30.00000, e = 0.01023, x = 29.98977, y = 36.58562

epoch = 479 w = 30.00000, e = 0.01025, x = 29.98975, y = 36.58596

epoch = 480 w = 30.00000, e = 0.01027, x = 29.98973, y = 36.58619

epoch = 481 w = 30.00000, e = 0.01027, x = 29.98973, y = 36.58630

epoch = 482 w = 30.00000, e = 0.01027, x = 29.98973, y = 36.58630

epoch = 483 w = 30.00000, e = 0.01027, x = 29.98973, y = 36.58619

epoch = 484 w = 30.00000, e = 0.01025, x = 29.98975, y = 36.58599

epoch = 485 w = 30.00000, e = 0.01023, x = 29.98977, y = 36.58568

epoch = 486 w = 30.00000, e = 0.01021, x = 29.98979, y = 36.58529

epoch = 487 w = 30.00000, e = 0.01017, x = 29.98983, y = 36.58480

epoch = 488 w = 30.00000, e = 0.01014, x = 29.98986, y = 36.58424

epoch = 489 w = 30.00000, e = 0.01009, x = 29.98991, y = 36.58362

epoch = 490 w = 30.00000, e = 0.01005, x = 29.98995, y = 36.58294

epoch = 491 w = 30.00000, e = 0.01000, x = 29.99000, y = 36.58221

epoch = 492 w = 30.00000, e = 0.00994, x = 29.99006, y = 36.58142

epoch = 493 w = 30.00000, e = 0.00989, x = 29.99011, y = 36.58060

epoch = 494 w = 30.00000, e = 0.00983, x = 29.99017, y = 36.57975

epoch = 495 w = 30.00000, e = 0.00977, x = 29.99023, y = 36.57888

epoch = 496 w = 30.00000, e = 0.00971, x = 29.99029, y = 36.57800

epoch = 497 w = 30.00000, e = 0.00965, x = 29.99035, y = 36.57713

epoch = 498 w = 30.00000, e = 0.00960, x = 29.99040, y = 36.57625

epoch = 499 w = 30.00000, e = 0.00954, x = 29.99046, y = 36.57541

*/

Listing 2: