12MHz CRYSTAL HEAD CAM PIPELINE

- ImageCapture

- MaskRectangle

300 x 300

- ConvertColor

Bgr2Gray

- MaskHsv

Hue: 20-120, Invert +, Saturation: 0-255, Value: 0-255

- Threshold

threshold="20"

- ImageRecall

- MaskHsv

Hue: 150-252, Saturation: 0-255, Value: 0-255

- Threshold

threshold="20"

- BlurMedian

kernel-size="7"

- Threshold

threshold="120"

- FindContours

Simple-List

- FilterContours

8000-20000

- MinAreaRectContours

- ImageRecall

- DrawContours

thickness="2" index="-1"> <color r="255" g="255" b="255" a="255"/>

- DrawRotatedRects

thickness="2" draw-rect-center="true" rect-center-radius="3" show-orientation="false"

SCRIPT

<cv-pipeline>

<stages>

<cv-stage class="org.openpnp.vision.pipeline.stages.ImageCapture" name="capture" enabled="true" settle-first="true"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.MaskRectangle" name="0" enabled="true" width="300" height="300"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.ConvertColor" name="gray" enabled="true" conversion="Bgr2Gray"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.MaskHsv" name="4" enabled="true" hue-min="20" hue-max="120" saturation-min="0" saturation-max="255" value-min="0" value-max="255" invert="true"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.Threshold" name="highlights" enabled="true" threshold="20" auto="false" invert="false"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.ImageRecall" name="5" enabled="true" image-stage-name="gray"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.MaskHsv" name="6" enabled="true" hue-min="150" hue-max="252" saturation-min="0" saturation-max="255" value-min="0" value-max="255" invert="false"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.Threshold" name="8" enabled="true" threshold="10" auto="false" invert="false"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.BlurMedian" name="merged" enabled="true" kernel-size="7"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.FindContours" name="contours" enabled="true" retrieval-mode="List" approximation-method="Simple"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.FilterContours" name="filtered_contours" enabled="true" contours-stage-name="contours" min-area="8000.0" max-area="20000.0"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.MinAreaRectContours" name="rects" enabled="true" contours-stage-name="filtered_contours"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.ImageRecall" name="recall2" enabled="true" image-stage-name="capture"/>

<cv-stage class="org.openpnp.vision.pipeline.stages.DrawContours" name="draw_contours" enabled="true" contours-stage-name="results" thickness="2" index="-1">

<color r="0" g="204" b="51" a="255"/>

</cv-stage>

<cv-stage class="org.openpnp.vision.pipeline.stages.DrawRotatedRects" name="draw_results" enabled="true" rotated-rects-stage-name="rects" thickness="2" draw-rect-center="true" rect-center-radius="3" show-orientation="true">

<color r="51" g="255" b="51" a="255"/>

</cv-stage>

</stages>

</cv-pipeline>